EE105: Introduction to Electrical Engineering

Module 1 Circuit Analysis

Objective:

In this lab, we will systematically analyze the circuits in Module 1 by applying Kirchhoff's Current Law and Ohm's Voltage Law. We will assign variables to voltages at each node and currents through each resistor, then formulate KCL and Ohm's law as a linear system of equations. Then, we will solve this system using Python's symbolic computation toolbox to obtain exact solutions.

Kirchhoff's Current Law (KCL): The algebraic sum of all currents entering and leaving a node (junction) in a circuit equals zero.

I1 + I2 + I3 =  I4 + I5

Ohm’s Voltage Law:  The voltage across a resistor is directly proportional to the current flowing through it.

Analysis of Networks on Module 1:

Checkpoint1: Write Kirchhoff's Current Law (KCL) equations

it = i1 + i3

…

Checkpoint2: Ohm's Law (Voltage) equations

i1 × r1 = s1 - s2

…

Checkpoint3: Solve these equations using SymPy

from sympy import symbols, Eq, solve

# Define the variables

i1, i2, i3, i4, i5, i6, i7, i8, i9, i10, i11, i12, it = symbols(

'i1 i2 i3 i4 i5 i6 i7 i8 i9 i10 i11 i12 it'

)

s1, s2, s3, s4, s5, s6, s7, s8, s9 = symbols('s1 s2 s3 s4 s5 s6 s7 s8 s9')

# assign resistance values

# TODO

# r1, ..., = ...

# Define the equations

# TODO:

# equations = [

#     Eq(it, i1 + i3),

#     ....

# ]

# Substitute s1 = 3.3, s9 = 0

substitutions = {s1: 3.3, s9: 0}

# Solve the system of equations

solution = solve(equations, (i1, i2, i3, i4, i5, i6, i7, i8, i9, i10, i11, i12, it, s2, s3, s4, s5, s6, s7, s8))

# Substitute the known values

solution_substituted = {k: v.subs(substitutions) for k, v in solution.items()}

solution_substituted

Additional SymPy Syntax:

Lt(x, y)   # equivalent to x < y  - Less than

Le(x, y)   # equivalent to x <= y - Less than or equal

Gt(x, y)   # equivalent to x > y  - Greater than

Ge(x, y)   # equivalent to x >= y - Greater than or equal

Ne(x, y)   # equivalent to x != y - Not equal

Checkpoint4: